Proposed revision to Methyl Bromide (MB) Fumigation Schedule of Annex I of

ISPM No. 15 (Guidelines for regulating wood packaging material in international trade.)

Comments:

These proposed changes are considered necessary due to concerns raised at various meetings of the ICPM about efficacy of the methyl bromide fumigation schedule in Annex I, Approved Measures Associated with Wood Packaging Material, of ISPM No. 15. (Guidelines for regulating wood packaging material in international trade.)

Existing text:

| Temperature | Dosage rate | Minimum concentration (g/m3) at: |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0.5hrs. | 2hrs. | 4hrs. | 16hrs. |
| 21oC or above | 48 | 36 | 24 | 17 | 14 |
| 16oC or above | 56 | 42 | 28 | 20 | 17 |
| 11oC or above | 64 | 48 | 32 | 22 | 19 |

The minimum temperature should not be less than 100C and the minimum exposure time should be 16 hours.

Proposed revision:

| Temperature | Dosage g/m3 | Minimum concentration (g/m3) at: |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 hrs. | 4 hrs. | 12 hrs. | 24 hrs. |
| 21o C or above | 48 | 36 | 31 | 28 | 24 |
| 16o C or above | 56 | 42 | 36 | 32 | 28 |
| 10o C or above | 64 | 48 | 42 | 36 | 32 |

The minimum temperature should not be less than 10o C and the minimum exposure time should be 24 hours.